SL-I (IOT with AI) List of experiments.

1.Write an application to read temperature from the environment. If temperature crosses threshold value then it notifies with buzzer.

const int sensorPin = A0;

const int buzzerPin = 8;

const float threshold = 40.0;

void setup() {

Serial.begin(9600);

pinMode(sensorPin, INPUT);

pinMode(buzzerPin, OUTPUT);

digitalWrite(buzzerPin, LOW);

}

void loop() {

int sensorValue = analogRead(sensorPin);

float voltage = sensorValue * (5.0 / 1023.0);

float temperature = voltage * 100.0;

Serial.print("Temperature: ");

Serial.print(temperature);

Serial.println(" °C");

if (temperature > threshold) {

digitalWrite(buzzerPin, HIGH);

} else {

digitalWrite(buzzerPin, LOW);

}

delay(1000);

}

2. Write a program using Arduino to control LED (One or more ON/OFF). Or Blinking.

const int led1 = 8;

const int led2 = 9;

const int led3 = 10;

void setup() {

pinMode(led1, OUTPUT);

pinMode(led2, OUTPUT);

pinMode(led3, OUTPUT);

}

void loop() {

digitalWrite(led1, HIGH);   // Turn ON LED 1

delay(1000);

digitalWrite(led1, LOW);    // Turn OFF LED 1

digitalWrite(led2, HIGH);   // Turn ON LED 2

delay(1000);

digitalWrite(led2, LOW);    // Turn OFF LED 2

digitalWrite(led3, HIGH);   // Turn ON LED 3

delay(1000);

digitalWrite(led3, LOW);    // Turn OFF LED 3

}

3. Create a program so that when the user enters ‘b’ the green light blinks, ‘g’ the green light is illuminated ‘y’ the yellow light is illuminated and ‘r’ the red light is illuminated.

const int greenLED = 13;

const int yellowLED = 8;

const int redLED = 5;

void setup() {

pinMode(greenLED, OUTPUT);

pinMode(yellowLED, OUTPUT);

pinMode(redLED, OUTPUT);

Serial.begin(9600);

Serial.println("Enter a command:");

Serial.println("'b' = Blink green LED");

Serial.println("'g' = Turn ON green LED");

Serial.println("'y' = Turn ON yellow LED");

Serial.println("'r' = Turn ON red LED");

}

void loop() {

if (Serial.available()) {

char command = Serial.read();

command = tolower(command);

if (command == 'b') {

Serial.println("Blinking green LED...");

for (int i = 0; i < 5; i++) {

digitalWrite(greenLED, HIGH);

delay(500);

digitalWrite(greenLED, LOW);

delay(500);

}

}

else if (command == 'g') {

Serial.println("Green LED ON");

digitalWrite(greenLED, HIGH);

digitalWrite(yellowLED, LOW);

digitalWrite(redLED, LOW);

}

else if (command == 'y') {

Serial.println("Yellow LED ON");

digitalWrite(greenLED, LOW);

digitalWrite(yellowLED, HIGH);

digitalWrite(redLED, LOW);

}

else if (command == 'r') {

Serial.println("Red LED ON");

digitalWrite(greenLED, LOW);

digitalWrite(yellowLED, LOW);

digitalWrite(redLED, HIGH);

}

else {

Serial.println("Invalid command! Use b, g, y, or r.");

}

}

}

4. Write a program that asks the user for a number and outputs the number squared that is entered.

int number;

void setup() {

Serial.begin(9600);

Serial.println("Enter a number to find its square:");

}

void loop() {

if (Serial.available() > 0) {

number = Serial.parseInt();

int square = number * number;

Serial.print("The square of ");

Serial.print(number);

Serial.print(" is: ");

Serial.println(square);

Serial.println();

Serial.println("Enter another number:");

delay(500);

}

}

5. Write a program to control the color of the LED by turning 3 different potentiometers. One will be read for the value of Red, one for the value of Green, and one for the value of Blue.

6. Write a program read the temperature sensor and send the values to the serial monitor on the computer.

const int lm35Pin = A0;

void setup() {

Serial.begin(9600);

}

void loop() {

int sensorValue = analogRead(lm35Pin);

float voltage = sensorValue * (5000.0 / 1023.0);

float temperatureC = voltage / 10.0;

if (temperatureC >= -50 && temperatureC <= 150) {

Serial.print("Temperature: ");

Serial.print(temperatureC);

Serial.println(" °C");

}

else {

Serial.println("Error: Unreasonable temperature reading. ");

}

delay(4000);

}

7. Write a program so it displays the temperature in Fahrenheit as well as the maximum and minimum temperatures it has seen.

const int sensorPin = A0;

float tempF;

float maxTemp = -1000;

float minTemp = 1000;

void setup() {

Serial.begin(9600);

Serial.println("Temperature Monitor (Fahrenheit)");

}

void loop() {

int sensorValue = analogRead(sensorPin);

float voltage = sensorValue * (5.0 / 1023.0);

float tempC = voltage * 100;

tempF = (tempC * 9.0 / 5.0) + 32.0;

if (tempF > maxTemp) maxTemp = tempF;

if (tempF < minTemp) minTemp = tempF;

Serial.print("Temp: ");

Serial.print(tempF);

Serial.print(" F  |  Max: ");

Serial.print(maxTemp);

Serial.print(" F  |  Min: ");

Serial.print(minTemp);

Serial.println(" F");

delay(2000);

}

8. Write a program to show the temperature and shows a graph of the recent measurements.

9. Write a program using piezo element and use it to play a tune after someone knocks.

10. Write an application using Raspberry Pi/Arduino for smart health monitoring system which records heart beat rate and temperature and also sends sms alerts if readings are beyond critical values.

11.Write an application for stopwatch or countdown timer.

int countdownTime = 10;

void setup() {

Serial.begin(9600);

Serial.println("Countdown Timer Started!");

}

void loop() {

for (int i = countdownTime; i >= 0; i--) {

Serial.print("Time Left: ");

Serial.print(i);

Serial.println(" seconds");

delay(1000);  // wait 1 second

}

Serial.println("Time's up!");

while (1);

}